Server Implementation & Infrastructure Requirements

Self-Hosted Jitsi Meet & Jibri Screen Recording (Migration from 8x8 JaaS)
Scenario: 1 Host (Screen Share + Audio) + 500 Attendees | Output: 1080p MP4

――――――――――――――――――――――――――――――――――――――――――――――――――――――――――

# 1. Target Workload & Traffic Overview

This specification outlines the server hardware, networking, kernel tuning, and configuration required to deploy a self-hosted Jitsi Meet cluster with dedicated Jibri server-side recording, replacing 8x8 JaaS on your dedicated platform.

1 Host (Presenter): Streams 1080p screen share (~3.0 Mbps), webcam (~1.5 Mbps), and audio (~48 kbps). Ingress to server is ~4.5 Mbps.

500 Attendees (Viewers): Connect in listen-only mode with camera and microphone muted. Each attendee receives the 1080p presentation stream. Outbound WebRTC egress from JVB peaks at 1.25 Gbps to 1.90 Gbps.

Jibri Recorder: Runs as a headless containerized participant with channelLastN=1 (ignoring the 500 attendees). It auto-pins the host screen share, capturing clean 1080p 30fps video and PulseAudio sound into an MP4 file.

# 2. Server Hardware Specifications

Because 500 concurrent participants downloading a 1080p screen share generate between 1.25 Gbps and 1.90 Gbps of outbound WebRTC UDP traffic, the server hosting Jitsi Videobridge (JVB) requires a dedicated 10 Gbps network interface.

## Option A: Recommended Single Bare-Metal Node (All-in-One)

All services (Nginx, Prosody, Jicofo, JVB, and Jibri) run on one high-performance bare-metal host (e.g., Hetzner AX102, OVH Advance, or AWS bare-metal).

| Hardware Component | Minimum Specification | Technical Purpose / Rationale |
| --- | --- | --- |
| Processor (CPU) | 16 Cores / 32 Threads (AMD EPYC, Ryzen 9, Intel Xeon) | Handles JVB WebRTC packet routing, Chromium headless rendering, and real-time x264 FFmpeg encoding. |
| System RAM | 64 GB DDR4/DDR5 ECC | Buffers 500 WebRTC peer connections and isolates Chromium browser execution. |
| Network Card (NIC) | 10 Gbps Port (Dedicated/Unmetered) | CRITICAL: Outbound WebRTC traffic to 500 users is 1.3 - 2.0 Gbps. Standard 1 Gbps ports will bottleneck. |
| Storage (OS & Apps) | 500 GB NVMe SSD (PCIe Gen4) | Fast system boot, container images, and temporary cache. |
| Storage (Recordings) | 1 TB NVMe SSD (Dedicated mount) | Dedicated sequential write buffer for 1080p MP4 recordings (~1.85 GB/hour). |
| Operating System | Ubuntu Server 22.04 or 24.04 LTS | Modern WebRTC socket support and current Docker Compose V2 runtime. |

## Option B: Distributed 3-Node Cluster

Splits the signaling, media routing, and video encoding across three specialized servers:

| Node Role | vCPU / RAM | Network Port | Assigned Workload |
| --- | --- | --- | --- |
| Node 1: Control Plane | 4 vCPU / 8 GB RAM | 1 Gbps NIC | Nginx Web SSL, Prosody XMPP (JWT Auth), and Jicofo. |
| Node 2: Media Bridge (JVB) | 16 vCPU / 32 GB RAM | 10 Gbps NIC (Crucial) | Routes WebRTC audio/video/screen packets to 500 users. |
| Node 3: Recording (Jibri) | 8 vCPU / 16 GB RAM | 1 Gbps NIC + 1TB NVMe | Runs headless Chrome, PulseAudio, and FFmpeg encoder. |

# 3. Container Resource Limits & Docker Shared Memory

To ensure system stability, configure CPU and RAM boundaries in docker-compose.yml and jibri.yml. Note that Jibri requires shm_size: '4gb' to prevent Chromium from crashing during 1080p decoding.

services: jvb: deploy: resources: limits: cpus: '12.0' memory: 16384M reservations: cpus: '4.0' memory: 4096M jibri: shm_size: '4gb' # CRITICAL: Chromium crashes without at least 2GB-4GB shared memory deploy: resources: limits: cpus: '4.0' memory: 8192M reservations: cpus: '2.0' memory: 4096M

# 4. Network Firewall & Port Matrix

| Port | Protocol | Accessibility | Purpose |
| --- | --- | --- | --- |
| 80 | TCP | Public (0.0.0.0/0) | HTTP Web redirection & Let's Encrypt renewal |
| 443 | TCP | Public (0.0.0.0/0) | HTTPS Web client, WebSockets, and BOSH |
| 10000 | UDP | Public (0.0.0.0/0) | WebRTC audio/video media streams via JVB (Mandatory) |
| 8443 | TCP | Internal / Docker | Internal Nginx reverse proxy loopback |
| 5222 | TCP | Internal / Docker | Prosody XMPP client signaling |
| 5347 | TCP | Internal / Docker | Prosody XMPP component protocol |
| 2222 | TCP | Internal / Docker | Jibri REST health check daemon |

# 5. Host Linux Kernel & Socket Buffer Tuning

A standard Linux kernel drops UDP packets when routing media to >100 participants. Deploy the following configuration to /etc/sysctl.d/99-jitsi-500users.conf and apply with 'sudo sysctl --system':

# Socket receive & send buffer maximums for WebRTC net.core.rmem_max = 67108864 net.core.wmem_max = 67108864 net.core.rmem_default = 33554432 net.core.wmem_default = 33554432 # Buffer memory allocation for UDP queues net.ipv4.udp_mem = 65536 131072 262144 # System-wide file descriptor limit for 500+ WebRTC peer sockets fs.file-max = 2097152 # Ephemeral port range expansion net.ipv4.ip_local_port_range = 1024 65535 # Backlog queue limits net.core.netdev_max_backlog = 100000 net.core.somaxconn = 65535

In /etc/security/limits.conf, expand open file limits:

* soft nofile 65535 * hard nofile 65535 root soft nofile 65535 root hard nofile 65535

# 6. Step-by-Step Server Setup Runbook

## Step 1: Install Docker & Docker Compose V2

Execute the following commands on the Ubuntu/Debian server:

sudo apt update && sudo apt upgrade -y sudo apt install -y curl gnupg lsb-release apt-transport-https ca-certificates sudo mkdir -p /etc/apt/keyrings curl -fsSL https://download.docker.com/linux/ubuntu/gpg ¦ sudo gpg --dearmor -o /etc/apt/keyrings/docker.gpg echo "deb [arch=$(dpkg --print-architecture) signed-by=/etc/apt/keyrings/docker.gpg] https://download.docker.com/linux/ubuntu $(lsb_release -cs) stable" ¦ sudo tee /etc/apt/sources.list.d/docker.list > /dev/null sudo apt update && sudo apt install -y docker-ce docker-ce-cli containerd.io docker-compose-plugin sudo systemctl enable --now docker

## Step 2: Configure Environment Variables (jitsi/.env)

Configure the public domain, ports, JWT authentication, and Jibri settings in jitsi/.env:

PUBLIC_URL=https://meet.yourplatform.com HTTP_PORT=80 HTTPS_PORT=443 JVB_PORT=10000 COMPOSE_FILE=docker-compose.yml:jibri.yml JIBRI_RECORDING_DIR=/storage/recordings ENABLE_RECORDING=1 ENABLE_SERVICE_RECORDING=1 JIBRI_BREWERY_MUC=jibribrewery JIBRI_RECORDER_USER=recorder JIBRI_RECORDER_PASSWORD=strong_recorder_password JIBRI_XMPP_USER=jibri JIBRI_XMPP_PASSWORD=strong_jibri_password # JWT Authentication matching your platform backend ENABLE_AUTH=1 AUTH_TYPE=jwt JWT_APP_ID=your_platform_app_id JWT_APP_SECRET=your_platform_app_secret JWT_ACCEPTED_ISSUERS=your_platform_backend JWT_ACCEPTED_AUDIENCES=jitsi CHROMIUM_FLAGS=--autoplay-policy=no-user-gesture-required,--use-fake-ui-for-media-stream,--start-maximized,--kiosk,--enabled,--host-resolver-rules=MAP meet.yourplatform.com web:8443

## Step 3: Configure Jibri for Host Screen Recording Only

To protect Jibri from 500 audience streams and guarantee clean slide capture, Jibri joins with conference URL parameters:

// Stream isolation: requests only 1 video stream (the host screen) config.channelLastN = 1; config.disableTileView = true; config.disableAutoPinOnScreenShare = false; // Clean broadcast: hides toolbars, popups, and badges from MP4 config.hideConferenceSubject = true; config.hideConferenceTimer = true; config.disableNotifications = true; config.disableChat = true; interfaceConfig.TOOLBAR_BUTTONS = []; interfaceConfig.SHOW_JITSI_WATERMARK = false;

## Step 4: Update Dedicated Platform Frontend (Replacing JaaS)

In your platform web application, update the Jitsi IFrame / React initialization:

const domain = "meet.yourplatform.com"; const options = { roomName: "webinar-hall-500", width: "100%", height: "100%", parentNode: document.querySelector("#jitsi-container"), jwt: clientBackendGeneratedJWT, configOverwrite: { startWithAudioMuted: !isHost, // 500 attendees start muted startWithVideoMuted: !isHost, // 500 attendees start with camera off channelLastN: 2, // Viewers receive screen share + presenter cam disableAutoPinOnScreenShare: false }, interfaceConfigOverwrite: { TOOLBAR_BUTTONS: isHost ? ['microphone', 'camera', 'desktop', 'recording', 'chat', 'participants-pane', 'hangup'] : ['chat', 'raisehand', 'hangup'] } }; const api = new JitsiMeetExternalAPI(domain, options);

## Step 5: Backend JWT Token Generation (Host vs. Viewers)

// Host Presenter Token Payload: { "context": { "user": { "name": "Host Presenter", "moderator": true }, "features": { "recording": true } }, "aud": "jitsi", "iss": "your_platform_backend", "sub": "meet.yourplatform.com", "room": "webinar-hall-500" } // 500 Attendees Token Payload: { "context": { "user": { "name": "Attendee", "moderator": false }, "features": { "recording": false } }, "aud": "jitsi", "iss": "your_platform_backend", "sub": "meet.yourplatform.com", "room": "webinar-hall-500" }

## Step 6: Start Services & Health Checks

cd jitsi # 1. Start all containers in background docker compose up -d # 2. Check running status (web, prosody, jicofo, jvb, jibri) docker compose ps # 3. Check Jibri health docker exec jitsi-jibri-1 curl -s http://127.0.0.1:2222/jibri/api/v1.0/health # Response: {"busyStatus":"IDLE","health":{"healthStatus":"HEALTHY"}} # 4. Follow live recording logs when host starts recording docker compose logs -f jibri